36 CCNA PRACTICAL LABORATORIES

Knowledge consolidation — from basics to final exam

Packet Tracer | Ranked from simplest to most complex

Prepared by Rigers Maja

# How to use this summary

These 36 labs are ranked by progressive difficulty and grouped into 8 modules. Each lab has a clear objective, topology, step-by-step requirements, and verification commands. Methodology: you solve it yourself in Packet Tracer, then send me 'show running-config' of the devices + verification results, and I correct them explaining any errors.

MODULE 1: BASICS — IP, CONNECTIVITY, INITIAL CONFIGURATION

## Lab 1: Basic switch configuration

Difficulty: Easy

Objective: Configure hostname, banner, passwords, and SSH on a switch.

Topology: 1 Switch (2960) + 1 PC connected with console cable.

Requirements:

- Change the hostname to SW1.

- Set MOTD banner: 'Authorized Access Only'.

- Configure enable secret: 'class' and console password: 'cisco'.

- Configure VTY lines 0-15 with password 'cisco' and transport input ssh.

- Create domain name 'lab.local' and generate 1024-bit RSA key.

- Create admin user with secret 'admin123' and set 'login local' to VTY.

- Save the configuration with 'copy run start'.

Verification (send these outputs):

- show running-config

- show ip ssh

- show users

## Lab 2: Basic router configuration

Difficulty: Easy

Objective: Configure hostname, interfaces, and IP addressing on a router.

Topology: 1 Router (2911) + 2 PCs (each on different interfaces).

Requirements:

- Change the hostname to R1.

- Configure G0/0 with IP 192.168.1.1/24 and G0/1 with IP 192.168.2.1/24.

- Enable both interfaces (no shutdown).

- Set PC1 with IP 192.168.1.10/24 (gateway .1) and PC2 with IP 192.168.2.10/24 (gateway .1).

- Set MOTD, enable secret, console/VTY passwords as Lab 1.

- Test ping from PC1 to PC2.

Verification (send these outputs):

- show ip interface brief

- show running-config

- ping from PC1 → PC2 (screenshot)

## Lab 3: Subnetting with VLSM

Difficulty: Easy

Objective: Divide a /24 network into different subnets with VLSM.

Topology: 1 Router (2911) + 3 segments: LAN A (60 hosts), LAN B (30 hosts), LAN C (10 hosts). Base network: 10.0.0.0/24.

Requirements:

- Calculate the VLSM subnets on paper: LAN A (/26), LAN B (/27), LAN C (/28).

- Configure the 3 router interfaces with the first IP of each subnet.

- Set PCs with correct IPs within each subnet.

- Test ping between segments.

Verification (send these outputs):

- show ip interface brief

- VLSM table on paper (photo/text)

- ping between LANs

## Lab 4: Basic VLAN

Difficulty: Easy

Objective: Create VLANs and assign ports to them.

Topology: 1 Switch (2960) + 4 PCs (2 in VLAN 10, 2 in VLAN 20).

Requirements:

- Create VLAN 10 named 'Sales' and VLAN 20 named 'IT'.

- Assign Fa0/1, Fa0/2 to VLAN 10 (access mode).

- Assign Fa0/3, Fa0/4 to VLAN 20 (access mode).

- Set IPs: VLAN10 PCs 10.0.10.x/24, VLAN20 PCs 10.0.20.x/24.

- Testo: ping within VLAN should work, between VLANs not.

Verification (send these outputs):

- show vlan brief

- show switchport interfaces (Fa0/1 and Fa0/3)

- ping test

## Lab 5: Trunking between switches

Difficulty: Easy

Objective: Configure a trunk link between two switches to carry VLAN traffic.

Topology: 2 Switches (2960) + 4 PCs (2 in VLAN 10, 2 in VLAN 20, distributed across switches).

Requirements:

- On both switches, create VLAN 10 (Sales) and VLAN 20 (IT).

- SW1: Fa0/1 → VLAN 10, Fa0/2 → VLAN 20.

- SW2: Fa0/1 → VLAN 10, Fa0/2 → VLAN 20.

- Connection between switches (Fa0/24): configure as trunk (switchport mode trunk).

- Set allowed VLANs on trunk: only 10 and 20.

- Testo: PC in VLAN 10 of SW1 needs to ping PC in VLAN 10 of SW2.

Verification (send these outputs):

- show interfaces trunk

- show vlan brief (both switches)

- ping tests cross-switch within the VLAN

MODULE 2: SWITCHING — INTER-VLAN, STP, NATIVE VLAN

## Lab 6: Router-on-a-Stick (Inter-VLAN Routing)

Difficulty: Medium

Objective: Configure inter-VLAN routing using subinterfaces on the router.

Topology: 1 Router (2911) + 1 Switch (2960) + 3 PCs (VLAN 10, 20, 30).

Requirements:

- On the switch: create VLANs 10, 20, 30. Assign ports to PCs.

- Switch→router connection (Fa0/24): configure as trunk.

- On router G0/0: create subinterfaces .10, .20, .30.

- Each subinterface: encapsulation dot1Q <vlan-id>, IP gateway of the VLAN.

- Enable G0/0 (no shutdown — subinterfaces inherit).

- For PCs, set the gateway = IP of the corresponding subinterface.

- Test ping between VLANs.

Verification (send these outputs):

- show ip interface brief

- show vlans (router)

- show interfaces trunk (switch)

- ping between VLANs

## Lab 7: Inter-VLAN with SVI (Layer 3 Switch)

Difficulty: Medium

Objective: Configure inter-VLAN routing with SVI on a multilayer switch.

Topology: 1 L3 Switch (3560) + 4 PCs (VLAN 10, 20, 30, 99-management).

Requirements:

- Enable 'ip routing' on the switch.

- Create VLANs 10, 20, 30, 99.

- Create SVI for each VLAN (interface vlan X) with gateway IP.

- Assign access ports to PCs.

- VLAN 99: management VLAN — assign a port to the admin PC.

- Test ping between all VLANs.

Verification (send these outputs):

- show ip interface brief

- show ip route

- show vlan brief

- cross-VLAN ping

## Lab 8: STP Verification & Root Bridge

Difficulty: Medium

Objective: Verify STP, identify root bridge, and manipulate priority.

Topology: 3 Switches (2960) connected in a triangle topology.

Requirements:

- Connect switches: SW1↔SW2 (Fa0/1), SW2↔SW3 (Fa0/1), SW1↔SW3 (Fa0/2).

- Without changing anything, verify which switch is the root bridge.

- Identify which ports are Root, Designated, Blocked.

- Change the priority of SW1 to 4096 to force it to become the root bridge.

- Verify again — SW1 should be root now.

Verification (send these outputs):

- show spanning-tree (all 3 switches, before and after change)

- RP/DP/BP port diagram

## Lab 9: Native VLAN mismatch & Security

Difficulty: Medium

Objective: Identify and resolve native VLAN mismatch, configure secure native VLAN.

Topology: 2 Switches (2960) with trunk link + 2 PCs.

Requirements:

- Configure trunk between switches.

- On SW1 set native VLAN 99, on SW2 leave default (VLAN 1).

- Verify — you should see CDP error / native VLAN mismatch.

- Fix: set native VLAN 99 on both sides.

- Create VLAN 99 on both switches (without the VLAN created, traffic drops).

- Verify that the trunk is working without error.

Verification (send these outputs):

- show interfaces trunk (both switches)

- show spanning-tree (check native VLAN)

- show cdp neighbors

## Lab 10: Rapid PVST+ and PortFast

Difficulty: Medium

Objective: Configure Rapid PVST+ and optimize convergence with PortFast/BPDU Guard.

Topology: 2 Switches (2960) + 2 PCs.

Requirements:

- Change STP mode to rapid-pvst (spanning-tree mode rapid-pvst).

- On access ports (where PCs connect): enable PortFast.

- On access ports: enable BPDU Guard.

- Verify that the PCs switch directly to forwarding (without 30s delay).

- Test: connect another switch to the port with BPDU Guard — it should do err-disable.

Verification (send these outputs):

- show spanning-tree summary

- show spanning-tree interface Fa0/1 detail

- show interfaces status err-disabled

MODULE 3: ETHERCHANNEL & L2 REDUNDANCY

## Lab 11: LACP EtherChannel

Difficulty: Medium

Objective: Configure LACP EtherChannel between two switches.

Topology: 2 Switches (2960) with 2 physical connections between them + 2 PCs.

Requirements:

- Connect SW1↔SW2 to Fa0/1 and Fa0/2.

- On SW1: configure channel-group 1 mode active (Fa0/1-2).

- On SW2: configure channel-group 1 passive mode (Fa0/1-2).

- Configure Port-Channel 1 as trunk with allowed VLANs 10, 20.

- Create VLANs and assign PCs.

- Verify that the EtherChannel is Up.

Verification (send these outputs):

- show etherchannel summary

- show etherchannel port-channel

- show interfaces trunk

## Lab 12: PAgP EtherChannel

Difficulty: Medium

Objective: Configure PAgP (Cisco proprietary) EtherChannel.

Topology: 2 Switches (2960) with 2 physical connections.

Requirements:

- Connect SW1↔SW2 to Fa0/3 and Fa0/4.

- SW1: channel-group 2 mode desirable.

- SW2: channel-group 2 auto mode.

- Configure Po2 as a trunk.

- Verify that Po2 is Up and the protocol is PAgP.

Verification (send these outputs):

- show etherchannel summary

- show etherchannel protocol

## Lab 13: L3 EtherChannel

Difficulty: Medium

Objective: Configure EtherChannel with IP (Layer 3) between two L3 switches.

Topology: 2 L3 Switches (3560) with 2 physical connections.

Requirements:

- On both switches: no switchport on Fa0/1-2.

- Create channel-group 1 mode active (LACP).

- On Po1: set IP (10.0.0.1/30 on SW1, 10.0.0.2/30 on SW2).

- Enable IP routing.

- Test ping between switches over EtherChannel.

Verification (send these outputs):

- show etherchannel summary

- show ip interface brief

- ping between switches

## Lab 14: EtherChannel troubleshooting

Difficulty: Medium

Objective: Diagnose and correct an EtherChannel that is not forming.

Topology: 2 Switches (2960). Intentionally misconfigure — then correct.

Requirements:

- Connects 2 ports between switches.

- SW1: active mode, SW2: desirable mode (LACP vs PAgP — doesn't work!).

- Verify — EtherChannel should not be formed. Explain WHY.

- Correct: set SW2 to active or passive mode.

- Verify that the EtherChannel is formed.

- Mistake #2: Set different speeds on ports (100 vs auto). Verify the effect.

- Correct the speed and verify.

Verification (send these outputs):

- show etherchannel summary (before and after correction)

- Explanation of the causes

## Lab 15: Multi-VLAN trunk over EtherChannel

Difficulty: Medium

Objective: Combine VLAN, trunk, EtherChannel, and SVI routing.

Topology: 2 L3 Switches (3560) + 4 PCs (VLAN 10, 20 distributed).

Requirements:

- Create LACP EtherChannel (Po1) between switches, configure as trunk.

- Create VLANs 10 and 20 on both switches.

- On SW1: create SVIs for VLAN 10 (10.0.10.1/24) and VLAN 20 (10.0.20.1/24).

- Enable ip routing on SW1 (SW1 becomes the default gateway).

- Assign PCs: 2 to VLAN 10 (one per switch), 2 to VLAN 20.

- Cross-VLAN and cross-switch ping test.

Verification (send these outputs):

- show etherchannel summary

- show ip route (SW1)

- show vlan brief

- ping cross-VLAN cross-switch

MODULE 4: ROUTING — STATIC, DEFAULT, OSPF

## Lab 16: Static Routes

Difficulty: Medium

Objective: Configure static routes between 3 routers.

Topology: 3 Routers (2911) in series: R1—R2—R3. Each router has a LAN.

Requirements:

- R1 LAN: 192.168.1.0/24, R2 LAN: 192.168.2.0/24, R3 LAN: 192.168.3.0/24.

- WAN R1↔R2: 10.0.12.0/30, WAN R2↔R3: 10.0.23.0/30.

- Configure static routes so that each LAN can reach every other LAN.

- Don't use default routes — only specific static routes.

- Connect 1 PC to each LAN and test end-to-end ping.

Verification (send these outputs):

- show ip route (all 3 routers)

- ping PC1→PC3

## Lab 17: Default Route and Gateway of Last Resort

Difficulty: Medium

Objective: Configure default route to ISP.

Topology: 2 Routers: R1 (internal) + ISP. R1 has two LANs.

Requirements:

- R1 LAN1: 192.168.10.0/24, LAN2: 192.168.20.0/24.

- WAN R1↔ISP: 203.0.113.0/30.

- On R1: configure default route (0.0.0.0/0) towards ISP next-hop.

- At the ISP: configure static return routes to the R1 LANs.

- Verify 'Gateway of last resort' on R1.

Verification (send these outputs):

- show ip route (R1 — verify * flag)

- ping from LAN to ISP loopback

## Lab 18: Floating Static Route

Difficulty: Medium

Objective: Configure backup link with floating static route (higher AD).

Topology: 2 Routers with 2 connections: primary (Fa) + backup (Serial).

Requirements:

- Primary link: 10.0.0.0/30 (G0/0↔G0/0).

- Backup link: 10.0.1.0/30 (S0/0/0↔S0/0/0).

- On both routers: static route to the other LAN with next-hop primary.

- Add floating static route with AD=10 through the backup link.

- Verify: only the primary route should appear in the routing table.

- When the primary link is down (shutdown) — floating route should be activated automatically.

- Primary restore (no shutdown) — the original route should be restored.

Verification (send these outputs):

- show ip route (before shutdown, after shutdown, after no shutdown)

- ping test during failover

## Lab 19: Basic OSPF Single-Area

Difficulty: Medium

Objective: Configure single-area OSPF between 3 routers.

Topology: 3 Routers in a triangle. Each router has 1 LAN.

Requirements:

- R1 LAN: 10.1.1.0/24, R2 LAN: 10.2.2.0/24, R3 LAN: 10.3.3.0/24.

- WAN links: 10.0.12.0/30, 10.0.23.0/30, 10.0.13.0/30.

- Configure OSPF process 1, area 0 on all 3 routers.

- Use network statements with correct wildcard masks.

- Set the router-id manually: 1.1.1.1, 2.2.2.2, 3.3.3.3.

- Passive-interface on LAN interfaces.

Verification (send these outputs):

- show ip ospf neighbor

- show ip ospf interface brief

- show ip route ospf

- end-to-end ping

## Lab 20: OSPF cost manipulation & DR/BDR

Difficulty: Hard

Objective: Manipulate OSPF cost and verify DR/BDR election.

Topology: 3 Routers + 1 multi-access segment (Ethernet).

Requirements:

- Configure OSPF as Lab 19.

- In the multi-access segment, identify which router was elected DR and BDR.

- Change OSPF priority: R1=100 (enforced DR), R2=50 (BDR), R3=0 (never DR).

- Do clear ip ospf process for re-election.

- Change OSPF cost on link R1↔R3 (ip ospf cost 100) — verify routing table change.

- Configure reference bandwidth: auto-cost reference-bandwidth 10000.

Verification (send these outputs):

- show ip ospf neighbor (DR/BDR roles)

- show ip ospf interface (cost values)

- show ip route ospf

## Lab 21: OSPF + Default Route Propagation

Difficulty: Hard

Objective: Propagate default route via OSPF to the internal network.

Topology: 3 Routers: R1, R2 (edge → ISP), R3. OSPF area 0.

Requirements:

- Configure OSPF between R1, R2, R3 (like Lab 19).

- R2 has a default route to the ISP (static 0.0.0.0/0).

- On R2: default-information originate.

- Verify: R1 and R3 should have O*E2 default route.

- Testo: PC from R1's LAN needs to reach ISP loopback.

- Bonus: try 'default-information originate always' — what changes?

Verification (send these outputs):

- show ip route ospf (R1 and R3 — search O*E2)

- show ip ospf database (external LSA)

- end-to-end ping

MODULE 5: SERVICES — DHCP, NAT/PAT

## Lab 22: DHCP Server on Router

Difficulty: Medium

Objective: Configure the router as a DHCP server.

Topology: 1 Router (2911) + 1 Switch + 4 PCs.

Requirements:

- Create DHCP pool 'LAN10' for network 192.168.10.0/24.

- Default gateway: .1, DNS: 8.8.8.8.

- Exclude addresses .1 to .10 (ip dhcp excluded-address).

- Set the PCs to DHCP (not static).

- Verify that the PCs obtain IP automatically.

Verification (send these outputs):

- show ip dhcp binding

- show ip dhcp pool

- ipconfig /all from PC

## Lab 23: DHCP Relay (ip helper-address)

Difficulty: Medium

Objective: Configure DHCP relay when the DHCP server is on a different subnet.

Topology: 2 Routers + 1 Switch. R1=DHCP server, R2=relay, PCs behind R2.

Requirements:

- R1: DHCP pool for the subnet of PCs behind R2.

- WAN R1↔R2: 10.0.0.0/30.

- PC subnet: 192.168.20.0/24 (behind R2).

- R2 G0/1 (towards PCs): ip helper-address 10.0.0.1 (IP of R1).

- Set the PCs to DHCP.

- Verify: PCs should get IP from R1's pool.

Verification (send these outputs):

- show ip dhcp binding (R1)

- ipconfig /all (PC)

- debug ip dhcp server events (optional)

## Lab 24: Static NAT and Dynamic NAT

Difficulty: Medium

Objective: Configure static NAT for the server and dynamic NAT for the PCs.

Topology: 1 Router + 1 Switch (inside) + ISP router (outside). 1 Server + 2 PCs.

Requirements:

- Inside network: 192.168.1.0/24. Server: .100.

- Outside (ISP): 203.0.113.0/30.

- Static NAT: map server .100 → 203.0.113.10 (permanent).

- Dynamic NAT pool: 203.0.113.20 - .30 for PCs.

- Create ACL 1 that allows 192.168.1.0/24.

- ip nat inside source list 1 pool NATPOOL.

- Set interfaces: ip nat inside / ip nat outside.

Verification (send these outputs):

- show ip nat translations

- show ip nat statistics

- ping from server/PC to ISP

## Lab 25: PAT (NAT Overload)

Difficulty: Medium

Objective: Configure PAT using the outside interface.

Topology: 1 Router + 1 Switch + 3 PCs + ISP router.

Requirements:

- Inside: 192.168.1.0/24.

- Outside: 203.0.113.0/30.

- ACL 1: permit 192.168.1.0 0.0.0.255.

- ip nat inside source list 1 interface G0/1 overload.

- Set ip nat inside/outside.

- Test: all 3 PCs must reach the ISP simultaneously.

- Verify translations — they should all have the same outside IP but different ports.

Verification (send these outputs):

- show ip nat translations

- show ip nat statistics

- simultaneous ping from 3 PCs

MODULE 6: SECURITY — ACL, PORT SECURITY

## Lab 26: Standard ACL

Difficulty: Medium

Objective: Configure standard ACLs to restrict network access.

Topology: 2 Routers + 2 LANs. LAN A: 192.168.1.0/24, LAN B: 192.168.2.0/24.

Requirements:

- Create standard ACL 10: deny host 192.168.1.10, permit any.

- Apply the ACL as close to the destination as possible (R2 interface towards LAN B, direction out).

- Testo: PC 192.168.1.10 should NOT reach LAN B. Other PCs should.

- Bonus: rewrite as named ACL 'BLOCK-HOST'.

Verification (send these outputs):

- show access-lists

- show ip interface (verify ACL applied)

- ping test

## Lab 27: Extended ACL

Difficulty: Hard

Objective: Configure extended ACL with port-specific filtering.

Topology: 1 Router + 2 LANs + 1 Server (HTTP/HTTPS).

Requirements:

- Server: 192.168.2.100 (web server).

- Create extended ACL 100:

- - Permit VLAN 10 (192.168.1.0/24) → server only HTTP (80) and HTTPS (443).

- - Deny VLAN 10 → server everything else.

- - Permit all other traffic.

- Apply ACL as close to the source as possible (direction in).

- Testo: browsing should work, ping/telnet/ssh should not.

Verification (send these outputs):

- show access-lists (hit counters)

- show ip interface

- HTTP vs ping vs telnet test

## Lab 28: Port Security

Difficulty: Medium

Objective: Configure port security to restrict MAC addresses.

Topology: 1 Switch (2960) + 2 PCs.

Requirements:

- Fa0/1: switchport port-security, maximum 1, violation shutdown.

- Switchport port-security mac-address sticky.

- Connect PC1 → learn MAC address (sticky).

- Disconnect PC1, connect PC2 to Fa0/1 → the port should become err-disabled.

- Restore: errdisable recovery cause psecure-violation, interval 30.

- Or manually: shutdown → no shutdown.

Verification (send these outputs):

- show port-security interface Fa0/1

- show port-security address

- show interfaces status err-disabled

## Lab 29: ACL + NAT + DHCP combined

Difficulty: Hard

Objective: Realistic scenario: combine DHCP, PAT, and ACL in one topology.

Topology: 1 Router (edge) + 1 Switch + 3 PCs + ISP. LAN: 192.168.50.0/24.

Requirements:

- Router as DHCP server for LAN (pool .50-.100, gateway .1).

- PAT overload towards ISP.

- Extended ACL: allow HTTP/HTTPS to the internet, block telnet/SSH to the ISP.

- Apply ACL + NAT in the right direction.

- Testo: PCs receive DHCP, browse ISP, but cannot telnet/SSH to ISP.

Verification (send these outputs):

- show ip dhcp binding

- show ip nat translations

- show access-lists

- test suite (ping, HTTP, telnet)

MODULE 7: FHRP & AVAILABILITY

## Lab 30: Basic HSRP

Difficulty: Hard

Objective: Configure HSRP between two routers for gateway redundancy.

Topology: 2 Routers + 1 Switch + 2 PCs. LAN: 192.168.1.0/24.

Requirements:

- R1 G0/0: 192.168.1.2/24, R2 G0/0: 192.168.1.3/24.

- HSRP virtual IP: 192.168.1.1 (this is the PC's gateway).

- R1: standby 1 ip 192.168.1.1, priority 110, preempt.

- R2: standby 1 ip 192.168.1.1, priority 100, preempt.

- PCs set gateway 192.168.1.1.

- R1 must be Active (highest priority).

- Test failover: shutdown G0/0 on R1 → R2 takes the Active role.

Verification (send these outputs):

- show standby brief (both routers)

- show standby (detail)

- continuous ping during failover

## Lab 31: HSRP with Tracking

Difficulty: Hard

Objective: Configure HSRP tracking — if the uplink goes down, priority is automatically lowered.

Topology: 2 Routers + 1 Switch + ISP. R1 and R2 have uplinks to the ISP.

Requirements:

- Configure HSRP as Lab 30.

- R1: standby 1 track G0/1 decrement 20 (G0/1 = uplink to ISP).

- Testo: with G0/1 up, R1 Active (priority 110).

- Shutdown G0/1 (simulates uplink loss) → priority drops to 90 → R2 becomes Active.

- No shutdown G0/1 → R1 returns to Active (preempt).

Verification (send these outputs):

- show standby brief (before/after shutdown)

- show standby (verify tracked interface)

## Lab 32: HSRP + OSPF + NAT/PAT (dual-edge)

Difficulty: Hard

Objective: Dual-edge topology with HSRP, OSPF, and NAT.

Topology: 2 Routers (R1, R2 edge) + 1 L3 Switch + ISP. OSPF area 0 inside.

Requirements:

- OSPF between R1, R2, L3-SW.

- R1 and R2: default route to ISP + default-information originate.

- HSRP on VLAN gateway (virtual IP as default gateway).

- PAT overload on both routers.

- ACL: permit only HTTP/HTTPS outbound.

- Test: traffic passes normally; shutdown uplink of R1 → failover without loss.

Verification (send these outputs):

- show ip ospf neighbor

- show standby brief

- show ip nat translations

- show access-lists

- failover test

MODULE 8: INTEGRATIONS — FINAL EXAM

## Lab 33: Complete Enterprise LAN

Difficulty: Hard

Objective: Build an enterprise LAN with VLAN, EtherChannel, SVI, DHCP.

Topology: 2 L3 Switches + 2 Access Switches + 8 PCs (4 VLANs).

Requirements:

- VLAN 10 (Sales), 20 (IT), 30 (HR), 99 (Mgmt).

- LACP EtherChannel between L3 switches (Po1, trunk).

- Trunk links from L3 switches to access switches.

- SVI routing to SW-L3-1 (primary gateway).

- DHCP pools for VLANs 10, 20, 30 (excluding .1-.10).

- PortFast + BPDU Guard on access ports.

- Port Security (sticky, max 2) on access ports.

- Test DHCP + inter-VLAN + cross-switch.

Verification (send these outputs):

- show vlan brief (all switches)

- show etherchannel summary

- show ip dhcp binding

- show ip route

- show port-security

- ping matrix

## Lab 34: WAN + OSPF + NAT full topology

Difficulty: Hard

Objective: WAN topology with OSPF, NAT, ACL, DHCP.

Topology: 3 Routers (HQ, Branch, ISP) + 2 Switches + 4 PCs.

Requirements:

- HQ LAN: 10.10.0.0/16 (VLSM-shared for 2 VLANs).

- Branch LAN: 10.20.0.0/16.

- WAN: 172.16.0.0/30 links.

- OSPF area 0 between HQ↔Branch. HQ→ISP: default static route.

- HQ: default-information origin.

- PAT overload at HQ towards ISP.

- Extended ACL: block Branch → ISP telnet/SSH, allow HTTP/HTTPS.

- DHCP relay: HQ as DHCP server for Branch as well (ip helper on Branch router).

- Testo: Branch PC receives DHCP from HQ, browses ISP, but does not telnet to ISP.

Verification (send these outputs):

- show ip ospf neighbor

- show ip route ospf

- show ip nat translations

- show ip dhcp binding

- show access-lists

- full ping + HTTP + telnet test matrix

## Lab 35: Troubleshooting challenge

Difficulty: Hard

Objective: Ready topology with 8 hidden errors — find and correct.

Topology: 2 Routers + 2 Switches + 4 PCs. VLAN, trunk, OSPF, DHCP, ACL, NAT.

Requirements:

- You will get a running-config list with 8 intentional errors.

- Errors include: trunk mismatch, VLAN missing, OSPF network wrong, DHCP excluded error, logical ACL inverted, NAT inside/outside inverted, SVI no shutdown, default route missing.

- Identify any errors by reading the config.

- Write the correction (specific commands).

- Send: list of errors + corrective commands.

Verification (send these outputs):

- List of 8 mistakes with explanation

- Corrective commands for each

## Lab 36: Final Lab Challenge — Enterprise + WAN + Security + Wireless

Difficulty: Hard

Objective: The ultimate lab — integrates everything into a realistic enterprise scenario.

Topology: 2 L3 Switches + 2 Access Switches + 2 Edge Routers + ISP + WLC + LAP + 6+ PC/Laptop.

Requirements:

- VLSM: to 10.0.0.0/16 for 4 VLANs + 2 WAN + management + wireless.

- VLAN 10, 20, 30, 99 with trunk + LACP EtherChannel.

- SVI inter-VLAN routing, DHCP pools, ip helper-address.

- OSPF single-area between L3-SW, R1, R2.

- HSRP with tracking on VLAN gateway.

- R1, R2 → ISP: default routes + default-information originate.

- PAT overload on edge routers.

- Extended ACL: port-specific filters (HTTP/HTTPS only outbound).

- Port Security (sticky, max 2) + PortFast + BPDU Guard.

- Wireless: WLC + LAP, WLAN with WPA2-PSK, dedicated VLAN.

- Testo: DHCP, inter-VLAN, internet access, failover, wireless client.

Verification (send these outputs):

- show vlan brief

- show etherchannel summary

- show ip ospf neighbor

- show standby brief

- show ip nat translations

- show ip dhcp binding

- show access-lists

- show port-security

- Full test matrix: DHCP + inter-VLAN + internet + failover + wireless